Table of Contents

# Detection Theory for Nonlinear Chirp Spread Spectrum

A self-contained derivation of the mathematics behind this project: why LoRa’s standard demodulator requires a linear chirp, the correlation-theorem-based decoder that works for any chirp shape at FFT-like cost, why hyperbolic FM (HFM) is exactly self-similar under Doppler time-scaling, and the dual-edge frequency-tracking loop. Each result links to the code that implements it and the tests that verify it, so every equation here has a numerical check sitting next to it in the repository.

## Notation

| Symbol | Meaning |
| --- | --- |
| B | swept bandwidth (Hz) |
| T | symbol duration (s) |
| SF | spreading factor; M=2SF is the symbol alphabet size |
| N | samples per symbol, N=T⋅Fs |
| Fs | sample rate (Hz) |
| gu | normalized trajectory shape, u∈0,1, g0=0, g1=1 |
| ft | instantaneous frequency |
| ϕt | instantaneous phase, ϕt=2π0tfτ dτ |
| sn | the base (m=0) reference waveform, sn=ejϕn/Fs |
| τm | the cyclic sample shift encoding symbol m: τm=roundmN/M |
| Xk | the discrete Fourier transform (DFT) of xn |

## 1. The chirp model

Every waveform in this project is generated from one formula:

ft=f0+B⋅gt/T,  ϕt=2π0tfτ dτ,  st=ejϕt

Standard LoRa is the special case gu=u (a straight ramp — constant chirp rate). This project’s whole subject is what happens when g is something else: quadratic, sigmoid, hyperbolic, etc. Implementation: nlfsc_lora/chirp.py (base_frequency, base_waveform); the numerical phase integral is a discrete phase accumulator, ϕn=ϕn−1+2πfn/Fs.

Each of the M=2SF symbols is a cyclic time shift of the same base waveform:

smn=sn+τm mod N,  τm=roundmN/M

This is exactly how real LoRa encodes a symbol, and it is the fact everything below hinges on: there are not M independent waveforms, there is one waveform and M shifts of it.

## 2. Why the standard LoRa decoder needs a linear chirp

The standard trick — dechirp, then read one FFT bin — is: multiply the received signal by the conjugate of the reference, and look at what’s left.

dt = smt⋅st = ejϕt+τm⋅e−jϕt = e jϕt+τm−ϕt

This is a pure tone — a single spike in the FFT, the thing a frequency-domain peak-finder can read a bin index off of — exactly when its instantaneous frequency doesn’t depend on t:

ddtϕt+τm−ϕt = ft+τm−ft =? constant in t

Claim. ft+τ−ft is constant in t for every shift τ if and only if f is affine, ft=f0+kt.

Proof. (⇐) If ft=f0+kt, then ft+τ−ft=kt+τ−kt=kτ, independent of t. (⇒) If ft+τ−ft=cτ for all t, differentiate both sides with respect to t: f′t+τ−f′t=0 for every t,τ, so f′ is constant, so f is affine. ◼

So the FFT-bin shortcut is not a general property of chirps — it is a one-off algebraic coincidence of the straight line. For any curved g, the dechirped signal dt is itself a (smaller) chirp, not a tone: its FFT is smeared across many bins instead of concentrated in one, and there is no single bin whose index reliably encodes m.

Implementation and verification: nlfsc_lora/receiver.py::fft_demod; tests/test_receiver.py::test_fft_demod_fails_for_properly_embedded_hyperbolic_chirp confirms this fails for HFM specifically, not just approximately. A direct before/after picture — the dechirped instantaneous frequency staying flat for a linear chirp but sweeping for HFM, and the resulting FFT going from one sharp peak to a smeared double-hump — is in the project’s chat history and can be regenerated from nlfsc_lora/receiver.py::dechirp.

## 3. A decoder that works for any chirp: brute-force correlation

Since one FFT can’t be trusted for a curved g, the direct fallback is to correlate the received signal against every candidate symbol waveform and keep the best match:

scorem = n=0N−1smn rn = n=0N−1sn+τm mod N rn,  m=argmaxmscorem

This is correct for any g — implemented as nlfsc_lora/receiver.py::matched_filter_bank_demod — but it is M separate length-N sums: ONM work per received symbol.

## 4. The fast decoder: the correlation theorem

### 4.1 One function standing in for all M scores

Define the circular cross-correlation of the base waveform s and the received signal r, as a function of an arbitrary lag l (not just the M special lags τm):

Cl = n=0N−1sn rn−l mod N

Claim. scorem=Cτm for every m.

Proof. Substitute k=n+τm mod N (so n=k−τm mod N) into scorem:

scorem=k=0N−1sk rk−τm mod N=Cτm

which has the same magnitude as Cτm. ◼

So the M separate brute-force scores are just M particular samples of one function Cl. If C can be computed cheaply for every lag at once, decoding becomes a lookup instead of a search.

### 4.2 The correlation theorem

Let Sk and Rk be the DFTs of sn and rn:

Sk=n=0N−1sn e−j2πkn/N,  Rk=n=0N−1rn e−j2πkn/N

Theorem. Cl=IDFT{Sk Rk}l.

Proof. Take the DFT of C with respect to l and substitute m=n−l mod N:

DFT{C}k=l=0N−1n=0N−1sn rn−l mod Ne−j2πkl/N=n=0N−1snm=0N−1rm e−j2πkn−m/N=n​sn e−j2πkn/N⏟Sk⋅m​rm e−j2πkm/N=Sk Rk

Inverting the DFT gives the claim. ◼

This is a general fact about discrete signals — the same identity underlies fast convolution and cross-correlation everywhere in signal processing; it has nothing to do with chirps specifically. What makes it applicable here is section 4.1: the M candidates being cyclic shifts of one waveform is what lets a single correlation function stand in for all of them.

### 4.3 The decoder

m = argmaxm IDFT{Sk Rk}τm

Compute S once per reference (cache it — it never changes), compute R once per received symbol, multiply, inverse-transform, and read off the M positions τm. Implementation: nlfsc_lora/receiver.py::fft_correlation_demod.

corr = np.fft.ifft(base_fft * np.conj(np.fft.fft(rx)))   # C[l] for every l at once
m_hat = int(np.argmax(np.abs(corr[valid_lags])))          # look up the M known positions

Correctness. tests/test_receiver.py::test_fft_correlation_demod_matches_matched_filter_bank_exactly checks bit-for-bit agreement with the brute-force decoder (section 3) across every trajectory in the project, noiseless and under heavy noise — not merely similar accuracy, the identical decision on every trial.

### 4.4 Complexity

| Decoder | Cost | Works for nonlinear g? |
| --- | --- | --- |
| fft_demod (§2) | ONlogN | No |
| matched_filter_bank_demod (§3) | ONM | Yes |
| fft_correlation_demod (§4) | ONlogN | Yes |

At SF=7 (N=512, M=128): NM≈65,500 versus ∼3Nlog2N≈13,800 for the three transforms — matching the measured ∼​85× speedup at that configuration.

## 5. Two Doppler models, and why they give different answers

### 5.1 Narrowband: a constant frequency shift

rt=st ej2πfΔt

The correct-symbol correlation magnitude loss reduces to 0Tej2πfΔt dt — independent of g, since st=1 always (constant envelope). What differs between trajectories is confusability with the neighboring symbol hypothesis, which is a smaller, shape-dependent effect (examples/output/04_ser_vs_cfo.png).

### 5.2 Wideband: time-scaling, and HFM’s exact self-similarity

The physically exact Doppler model is time-scaling, not a frequency shift:

rt=sαt,  α=1±v/c

Hyperbolic FM (linear-period modulation) is the trajectory whose instantaneous frequency has 1/ft linear in t:

ft=flow1−βt,  ϕt=−2πflowβln1−βt

Theorem (exact self-similarity under time-scaling). For any α>0,

ϕαt=ϕt−Δ+c,  Δ=1−ααβ, c=−2πflowβlnα

Proof. The algebraic identity 1−βαt=α1−βt−Δ holds exactly for Δ=1−α/αβ (expand the right side: α−αβt+αβΔ=α−αβt+1−α=1−αβt, which is the left side). Taking −2πflowβln⋅ of both sides:

ϕαt=−2πflowβlnα+ln1−βt−Δ=−2πflowβlnα + ϕt−Δ  ◼

Exponentiating, sαt=ejc⋅st−Δ — a Doppler-scaled copy of an HFM waveform is, exactly, a constant-phase-rotated, time-shifted copy of the same waveform. A matched filter built for the unscaled s still recognizes it perfectly; the scaling only moves where the peak appears, not how tall it is. A linear chirp has no such identity — time-scaling a linear chirp changes its chirp rate outright, producing a genuinely different waveform, which is exactly why its matched-filter response degrades under scaling instead of merely shifting (examples/output/07_doppler_scale_tolerance.png shows this numerically: HFM loses ~1 dB of peak magnitude across a ±10% scale sweep versus $>$10 dB for a linear chirp).

This identity is exact only for the idealized, unbounded-domain continuous waveform. A real burst has a finite window [0,T), so the shifted copy’s support doesn’t line up perfectly with the original observation window at the edges — which is the real, measured, nonzero loss reported above, and also why channel.py::apply_doppler_scale zero-pads past the end of a finite burst rather than extrapolating (§5.2 of the code, not this paper). It is also why this single-waveform advantage does not fully survive being plugged into LoRa’s own fine-grained, M-ary cyclic-shift decoding (examples/output/08_lora_ser_vs_doppler_scale.png) — that decoder’s sensitivity to a Doppler-induced lag (generic to any trajectory) dominates before HFM’s magnitude-preservation advantage gets a chance to matter.

## 6. Dual-edge frequency tracking

### 6.1 Local frequency and rate from a phase fit

Over a short window of 2w+1 samples centered at index n0, fit the unwrapped phase to a cubic in the local sample index k:

∠rn0+k ≈ a+bk+ck2+dk3,  k=−w,…,w

Reading off the linear and quadratic coefficients gives the local instantaneous frequency and chirp rate at n0:

fn0=bFs2π,  fn0=2cFs22π

A cubic is exact for a quadratic trajectory (whose phase law is exactly cubic in t) and a good local approximation otherwise. Implementation: nlfsc_lora/local_rate.py::local_freq_and_rate, nlfsc_lora/afc.py::_edge_measurement.

### 6.2 Why the rate identifies position independent of CFO

A constant carrier offset adds a constant to ft but leaves ft untouched. So, given the decoded symbol m (hence the known, CFO-free expected rate fexp at position n0), a residual-CFO estimate falls out of comparing the measured frequency to what that symbol predicts:

CFO=fn0−fexpn0∣m

Measuring at two positions — near each end of the symbol — and weighting each by how well its measured rate matches fexp lets a clean measurement outvote one corrupted by the cyclic-shift phase discontinuity, which can only ever occur at one fixed position per symbol:

CFO=wA CFOA+wB CFOBwA+wB,  wi=1 fi−fexp,i/fexp,i+ϵ

Implementation and the specific verified case where one edge is corrupted and the other compensates: nlfsc_lora/afc.py::dual_edge_cfo_estimate, tests/test_afc.py::test_dual_edge_estimate_exact_given_correct_symbol_noiseless.

### 6.3 The tracking loop

A first-order exponential filter (the same structure as a classical AFC/PLL loop) accumulates the per-burst estimate into a slowly-updated correction:

CFOtrackedi+1=CFOtrackedi+γCFOi−CFOtrackedi

For a linearly drifting true CFO (rate ρ per burst), this loop has a textbook steady-state lag of ρ/γ — a control-theory type-1 velocity error, not merely noise jitter; push ρ too far relative to γ and the tolerance can be exceeded by the lag alone, independent of measurement noise. Implementation: nlfsc_lora/afc.py::AFCLoop.

## Summary

| Result | Section | Code | Test |
| --- | --- | --- | --- |
| FFT-bin demod requires linear f | §2 | receiver.py::fft_demod | test_receiver.py::test_fft_demod_fails_for_properly_embedded_hyperbolic_chirp |
| Brute-force correlation decoder | §3 | receiver.py::matched_filter_bank_demod | test_receiver.py::test_matched_filter_bank_exact_noiseless |
| Fast correlation-theorem decoder | §4 | receiver.py::fft_correlation_demod | test_receiver.py::test_fft_correlation_demod_matches_matched_filter_bank_exactly |
| Narrowband Doppler is trajectory-blind at the correct symbol | §5.1 | channel.py::apply_cfo | test_channel.py |
| HFM’s exact self-similarity under time-scaling | §5.2 | doppler.py, trajectories.py::hyperbolic | test_doppler.py::test_hyperbolic_more_doppler_scale_tolerant_than_linear |
| Dual-edge CFO estimation | §6.1-6.2 | afc.py::dual_edge_cfo_estimate | test_afc.py::test_dual_edge_estimate_exact_given_correct_symbol_noiseless |
| AFC tracking loop | §6.3 | afc.py::AFCLoop, run_afc_sequence | test_afc.py::test_tracking_survives_a_drift_that_exceeds_the_static_capture_range |

## References

- Vangelista, L. “Frequency Shift Chirp Modulation: The LoRa Modulation.” IEEE Signal Processing Letters, 2017.

- Woodward, P. M. Probability and Information Theory, with Applications to Radar. Pergamon Press, 1953. (The ambiguity function.)

- Kroszczyński, J. “Pulse Compression by Means of Linear-Period Modulation.” Proceedings of the IEEE, 1969. (Hyperbolic/linear-period FM.)

- Oppenheim, A. V., Schafer, R. W. Discrete-Time Signal Processing. (The discrete correlation theorem, §4.2 above.)

See DISSERTATION_OUTLINE.md for how these results map onto a dissertation structure, and README.md for how to regenerate every figure referenced here.